INTRENET OF THINGS (IPCC)

LAB MANUAL

| Ex. No: 1 | Develop a program to blink 5 LEDs back and forth. |
| --- | --- |

Aim: Develop a program to blink 5 LEDs back and forth.

Coding

void setup()

{

pinMode(13, OUTPUT); pinMode(12, OUTPUT); pinMode(11, OUTPUT); pinMode(10, OUTPUT); pinMode(9, OUTPUT);

}

void loop()

{

digitalWrite(13, HIGH);// Wait for 1000 millisecond(s)

delay(1000);

digitalWrite(13, LOW);

delay(1000);

digitalWrite(12, HIGH);

delay(1000);

digitalWrite(12, LOW);

delay(1000); digitalWrite(11, HIGH); delay(1000); digitalWrite(11, LOW); delay(1000); digitalWrite(10, HIGH); delay(1000); digitalWrite(10, LOW); delay(1000); digitalWrite(9, HIGH); delay(1000); digitalWrite(9, LOW); delay(1000);

}

| Ex. No: 2 | Develop a program to interface a relay with Arduino board. |
| --- | --- |

Aim: Develop a program to interface a relay with Arduino board.

# Circuit Diagram:

Arduino Code:

void setup()

{

pinMode(3, OUTPUT);

}

void loop()

{

digitalWrite(3, HIGH);

delay(1000); // Wait for 1000 millisecond(s)

digitalWrite(3, LOW);

delay(1000); // Wait for 1000 millisecond(s)

}

| Ex.No: 3 &11 | Develop a program to deploy an intrusion detection system using Ultrasonic and sound sensors. |
| --- | --- |
| Date: |  |

Aim: Develop a program to deploy an intrusion detection system using Ultrasonic and sound sensors

# PSUEDOCode :

// C++ code

//

int trigPin = 10; int echoPin = 9; long time;

float distance; void setup()

{

pinMode(trigPin, OUTPUT); // SETTING OUTPUT PIN pinMode(echoPin, INPUT); // SETTING INPUT PIN Serial.begin(9600); // INITIALISING THE COMMUNICATION

}

void loop()

{

digitalWrite(trigPin,LOW); delayMicroseconds(2);

// transmitting sound for 10 microseconds digitalWrite(trigPin, HIGH); delayMicroseconds(10);

digitalWrite(10, LOW);

// calculating distance time=pulseIn(echoPin , HIGH); Serial.print("time: "); Serial.println(time);

distance = time * 0.0343/2;

// Printing out the final output => distance Serial.print("Distance:"); Serial.println(distance);

}

| Ex. No: 4 | Develop a program to control a DC motor with Arduino board. |
| --- | --- |

AIM: Develop a program to control a DC motor with Arduino board.

# PSUEDO Code:

void setup()

{

pinMode(13, OUTPUT);

pinMode(12, OUTPUT);

}

void loop()

{

digitalWrite(13, HIGH);

delay(1000); // Wait for 1000 millisecond(s)

digitalWrite(12, LOW);

delay(1000); // Wait for 1000 millisecond(s)

digitalWrite(13, LOW);

delay(1000); // Wait for 1000 millisecond(s)

digitalWrite(12, HIGH);

delay(1000); // Wait for 1000 millisecond(s)

}

| Ex. No: 5 | Develop a program to deploy smart street light system using LDR sensor. |
| --- | --- |

AIM: Develop a program to deploy smart street light system using LDR sensor

.

# Arduino Code

// C++ code

//

int LDR_VAL = 0;

void setup()

{

pinMode(A0, INPUT);

Serial.begin(9600);

pinMode(8, OUTPUT);

}

void loop()

{

LDR_VAL = analogRead(A0);

Serial.println(LDR_VAL);

if (LDR_VAL > 500) {

digitalWrite(8, HIGH);

}

digitalWrite(8, LOW);

delay(10); // Delay a little bit to improve simulation performance

}

| Ex. No: 6 | Develop a program to classify dry and wet waste with the Moisture sensor (DHT22). |
| --- | --- |

Aim: Develop a program to classify dry and wet waste with the Moisture sensor .

# Arduino Code:

// declare variables for pins const int sensorpin = A0; const int sensorpower = 8; const int LED1 = 2;

const int LED2 = 3; const int LED3 = 4;

// variable for sensor reading int sensor;

const int delayTime = 1000;

// "wet" and "dry" thresholds - these require calibration int wet = 800;

int dry = 500;

void setup(){ // code that only runs once

// set pins as outputs pinMode(LED1,OUTPUT); pinMode(LED2,OUTPUT); pinMode(LED3,OUTPUT);

pinMode(sensorpower,OUTPUT);

// initialize serial communication Serial.begin(9600);

}

void loop(){ // code that loops forever

// power on sensor and wait briefly digitalWrite(sensorpower,HIGH); delay(10);

// take reading from sensor sensor = analogRead(sensorpin);

// turn sensor off to help prevent corrosion digitalWrite(sensorpower,LOW);

// print sensor reading Serial.println(sensor);

// If sensor reading is greater than "wet" threshold,

// turn on the blue LED. If it is less than the "dry"

// threshold, turn on the red LED. If it is in between

// the two values, turn on the yellow LED. if(sensor>wet){ digitalWrite(LED1,LOW); digitalWrite(LED2,LOW); digitalWrite(LED3,HIGH);

}

else if(sensor<dry){ digitalWrite(LED1,HIGH);

digitalWrite(LED2,LOW); digitalWrite(LED3,LOW);

}

else{ digitalWrite(LED1,LOW); digitalWrite(LED2,HIGH); digitalWrite(LED3,LOW);

}

// wait before taking next reading delay(delayTime);

}

| Ex. No: 7 | Develop a program to read the pH value of a various substances like milk, lime and water. |
| --- | --- |

Aim: Develop a program to read the pH value of a various substances like milk, lime and water.

# Arduino Code:

/*

# This sample code is used to test the pH meter V1.0. # Editor : YouYou

# Ver	: 1.0

# Product: analog pH meter # SKU	: SEN0161

*/

#define SensorPin A0	//pH meter Analog output to Arduino Analog Input 0

#define Offset 0.00	//deviation compensate #define LED 13

#define samplingInterval 20

#define printInterval 800

#define ArrayLenth 40	//times of collection

pHArray[ArrayLenth];

int

int pHArrayIndex=0; void setup(void)

{

//Store the average value of the sensor feedback

pinMode(LED,OUTPUT); Serial.begin(9600);

Serial.println("pH meter experiment!");	//Test the serial monitor

}

void loop(void)

{

static unsigned long samplingTime = millis(); static unsigned long printTime = millis();

static float pHValue,voltage;

if(millis()-samplingTime > samplingInterval)

{

voltage = pHValue =

pHArray[pHArrayIndex++]=analogRead(SensorPin); if(pHArrayIndex==ArrayLenth)pHArrayIndex=0;

avergearray(pHArray, ArrayLenth)*5.0/1024; 3.5*voltage+Offset;

samplingTime=millis();

}

if(millis() - printTime > printInterval)	//Every 800 milliseconds, print a numerical, convert the state of the LED indicator

{

Serial.print("Voltage:"); Serial.print(voltage,2); Serial.print("	pH value: ");

Serial.println(pHValue,2); digitalWrite(LED,digitalRead(LED)^1);

printTime=millis();

}

}

double avergearray(int* arr, int number){ int i;

int max,min; double avg; long amount=0; if(number<=0){

Serial.println("Error number for the array to avraging!/n");

return 0;

}

if(number<5){	//less than 5, calculated directly statistics

for(i=0;i<number;i++){

amount+=arr[i];

}

avg = amount/number; return avg;

}else{

if(arr[0]<arr[1]){

min = arr[0];max=arr[1];

}

else{

min=arr[1];max=arr[0];

}

for(i=2;i<number;i++){ if(arr[i]<min){

amount+=min;	//arr<min

min=arr[i];

}else {

if(arr[i]>max){

amount+=max;	//arr>max

max=arr[i];

}else{

amount+=arr[i]; //min<=arr<=max

}

}//if

}//for

avg = (double)amount/(number-2);

}//if return avg;

}

| Ex. No: 8 | Develop a program to detect the gas leakage in the surrounding environment. |
| --- | --- |

AIM: Develop a program to detect the gas leakage in the surrounding environment.

# Arduino Code:

int LED = A1;	//Red LED

int LED1 = A3;	//Green LED

int gas_pin = A0;		// For Gas Sensor int buzzer_pin = A2;	// For Buzzer

void setup()

{

Serial.begin(9600);

pinMode (buzzer_pin, OUTPUT); pinMode (gas_pin, INPUT);

}

void loop() {

float sensorValue,gas_pin;

sensorValue = analogRead(gas_pin); // read analog input pin 0

if(sensorValue >= 300)

{

digitalWrite(LED,HIGH); digitalWrite(LED1,LOW);

digitalWrite (buzzer_pin, HIGH);

//Serial.println(); Serial.print(sensorValue); Serial.println(" |SMOKE DETECTED|");

}

else

{

digitalWrite(LED,LOW); digitalWrite(LED1,HIGH);

digitalWrite (buzzer_pin, LOW); Serial.println(); Serial.println("Sensor Value: "); Serial.print(sensorValue);

//Serial.print(" |Safe Mode|");

}

delay(1000);

}

| Ex. No: 9 | Develop a program to demonstrate weather station readings using Arduino. |
| --- | --- |

Aim: Develop a program to demonstrate weather station readings using Arduino.

# Arduino Code:

#include <LiquidCrystal.h> // including LiquidCrystal Library

LiquidCrystal lcd(12,11,5,4,3,2); // Creating LC object where parameters: (RS,E,DB4,DB5,DB6,DB7)

void setup()

{

lcd.begin(16,2); // 16 Characters per line and 2 lines lcd.setCursor(4,0); // Positioning of the LCD cursor. col=4, row=0

lcd.print("Monitoring"); // Printing text to the LCD lcd.setCursor(7,1); // Positioning of the LCD cursor. col=7, row=1 lcd.print("System"); // Printing text to the LCD

delay(2000); // time delay

}

void loop()

{

lcd.clear(); // Clearing the LCD screen and positioning the cursor in the upper-left corner

float Voltage=analogRead(A0)*0.004882814; // Variable declartion and analog input pin select

float degrees = ( Voltage - 0.5 ) * 100; // Variable declartion and modification

lcd.setCursor(3,0); // Positioning of the LCD cursor. col=3, row=0 lcd.print("Temp:"); // Printing text to the LCD

lcd.setCursor(8,0); // Positioning of the LCD cursor. col=8, row=0 lcd.print(degrees); // Printing text to the LCD

if (degrees<10) // Checking variable value using if function

{

lcd.setCursor(4,1); // Positioning of the LCD cursor. col=4, row=1 lcd.print("-TOO COLD"); // Printing text to the LCD

}

else if(degrees<18 &&degrees>10 ) // Checking variable value using else if function

{

lcd.setCursor(4,1); // Positioning of the LCD cursor. col=4, row=1 lcd.print("-COLD"); // Printing text to the LCD

}

else if(degrees<30 &&degrees>18 ) // Checking variable value using else if function

{

lcd.setCursor(4,1); // Positioning of the LCD cursor. col=4, row=1 lcd.print("-Normal Temp"); // Printing text to the LCD

}

else if(degrees<45 &&degrees>30 ) // Checking variable value using else if function

{

lcd.setCursor(4,1); // Positioning of the LCD cursor. col=4, row=1 lcd.print("-Hot"); // Printing text to the LCD

}

else if(degrees>45 ) // Checking variable value using else if function

{

lcd.setCursor(4,1); // Positioning of the LCD cursor. col=4, row=1 lcd.print("-TOO HOT"); // Printing text to the LCD

}

delay(2000); // time delay

lcd.clear(); // Clearing the LCD screen and positioning the cursor in the upper-left int ldr=analogRead(A1); // Variable declartion and analog input pin select

lcd.setCursor(1,0); // Positioning of the LCD cursor. lcd.print("Intensity:"); // Printing text to the LCD lcd.setCursor(12,0); // Positioning of the LCD cursor. lcd.print(ldr); // Printing text to the LCD

if (ldr<230) // Checking variable value using if function

{

lcd.setCursor(4,1); // Positioning of the LCD cursor. lcd.print("-TOO BRIGHT"); // Printing text to the LCD

}

else if (ldr<460 && ldr>230 ) // Checking variable value using else if function

{

lcd.setCursor(2,1); // Positioning of the LCD cursor. lcd.print("-Medium Light"); // Printing text to the LCD

}

else if (ldr>460) // Checking variable value using else if function

{

lcd.setCursor(4,1); // Positioning of the LCD cursor. lcd.print("-TOO DARK"); // Printing text to the LCD

}

delay(2000); // time delay

lcd.clear(); // Clearing the LCD screen and positioning the cursor in the upper-left corner

}

| Ex. No: 12 | Develop a program to simulate interfacing with the keypad module to record the keystrokes. |
| --- | --- |

AIM: Develop a program to simulate interfacing with the keypad module to record the keystrokes.

CODE:

#include <Keypad.h>

const byte numRows= 4; //number of rows on the keypad const byte numCols= 4; //number of columns on the keypad

//keymap defines the key pressed according to the row and columns just as appears on the keypad

char keymap[numRows][numCols]=

{

{'1', '2', '3', 'A'},

{'4', '5', '6', 'B'},

{'7', '8', '9', 'C'},

{'*', '0', '#', 'D'}

};

//Code that shows the the keypad connections to the arduino terminals byte rowPins[numRows] = {9,8,7,6}; //Rows 0 to 3

byte colPins[numCols]= {5,4,3,2}; //Columns 0 to 3

//initializes an instance of the Keypad class

Keypad myKeypad= Keypad(makeKeymap(keymap), rowPins, colPins, numRows, numCols);

void setup()

{

Serial.begin(9600);

}

//If key is pressed, this key is stored in 'keypressed' variable

//If key is not equal to 'NO_KEY', then this key is printed out

//if count=17, then count is reset back to 0 (this means no key is pressed during the whole keypad scan process

void loop()

{

char keypressed = myKeypad.getKey(); if (keypressed != NO_KEY)

{

Serial.println(keypressed);

}

}